Name: Talha Ali

Part 1: Class C Subnetting Scenarios

Scenario 1: The Small Law Firm The firm uses the block 192.168.15.0/24 and needs exactly 4 subnets.

- A) Required Subnet Mask: To find the number of borrowed bits ($n$), we use the$2n≥subnets.We need 4 subnets, so $22= 4$. Borrowing 2 bits from the default /24 mask gives us a new CIDR of /26. The binary 11000000 in the last octet converts to a decimal mask of 255.255.255.192.

- B) Usable Hosts: The formula for usable hosts is 2h-2, where $h$ is the remaining host bits (zeroes). With 6 host bits left ($32 - 26 = 6$), the math is $26 - 2 = 64 - 2 =$ 62 usable hosts per subnet.

- C) 2nd Subnet (Associates) Details: The block size (magic number) is calculated by subtracting the interesting octet from 256 ($256 - 192 = 64$). The subnets increment by 64 (0, 64, 128, 192). For the 2nd subnet:

Network ID: 192.168.15.64

Usable IP Range: 192.168.15.65 to 192.168.15.126

Broadcast Address: 192.168.15.127

Scenario 2: The Medical Clinic The clinic uses the block 200.50.25.0/24 and needs a minimum of 6 subnets.

- A) Borrowed Bits & Total Subnets: Using $2n\ge 6$, we must borrow 3 network bits $23= 8$, which will create a total of 8 subnets.

- B) Subnet Mask: Borrowing 3 bits changes the mask from /24 to /27. The binary 11100000 in the last octet gives a decimal mask of 255.255.255.224.

- C) 1st Subnet (Reception) Details: The block size is $256 - 224 = 32$. The 1st subnet starts at 0.

Network ID: 200.50.25.0

Broadcast Address: 200.50.25.31

Scenario 3: The High School Computer Labs The school uses 192.168.100.0/24 and needs 4 subnets supporting at least 50 computers each.

- A) Subnet Mask: We solve for hosts first using 2h-2\ge50. We need 6 host bits ($2^6 – 2 = 62$). Leaving 6 bits for hosts means 26 bits are for the network ($32 - 6 = 26$). The mask is /26, or 255.255.255.192.

- B) Maximum Usable Hosts: With 6 host bits, the maximum number is $26– 2 =$ 62 usable hosts.

- C) 4th Subnet Range: The block size is $256 - 192 = 64$. Subnets increment by 64: 0, 64, 128, 192. The 4th subnet is 192.

Usable IP Range: 192.168.100.193 to 192.168.100.254

Scenario 4: The Coffee Shop Chain The shop uses 198.100.50.0/24 and needs the maximum subnets supporting at least 14 usable hosts.

- A) Efficient Subnet Mask: Using $2h- 2 \ge 14$, we need exactly 4 host bits ($24 – 2 = 14$). The remaining 28 bits are for the network ($32 - 4 = 28$). The CIDR is /28, and the decimal mask is 255.255.255.240.

- B) Total Subnets: We borrowed 4 bits from the original /24. Using $24$, this creates 16 total subnets.

- C) 3rd Subnet Network ID: The block size is $256 - 240 = 16$. The networks are 0, 16, 32. The 3rd subnet is 198.100.50.32.

Scenario 5: The E-Commerce Startup The startup uses 203.0.113.0/24 and needs to divide it into 12 distinct subnets.

- A) Borrowed Bits: Using $2n\ge 12$, we must borrow 4 bits ($24 = 16$).

- B) Usable IP Addresses: Borrowing 4 bits leaves 4 host bits. $24 - 2 =$ 14 usable IP addresses per squad.

- C) Last Subnet Details: The mask is 255.255.255.240, making the block size 16. The last (16th) subnet starts at $256 - 16 = 240$.

Network ID: 203.0.113.240

Usable IP Range: 203.0.113.241 to 203.0.113.254

Broadcast Address: 203.0.113.255

Part 2: Class B Subnetting Scenarios

Scenario 6: The Regional Hospital The hospital uses the Class B network 172.16.0.0/16 and needs 50 separate subnets.

- A) Borrowed Bits & Mask: Using $2n\ge 50$, we borrow 6 bits ($26 = 64$). Adding 6 bits to the /16 default gives a CIDR of /22. The 3rd octet uses 6 bits (11111100), making the decimal mask 255.255.252.0.

- B) Usable Hosts: The mask is /22, leaving 10 host bits ($32 - 22 = 10$). $210 - 2 =$ 1,022 usable hosts.

- C) 2nd Subnet Details: The block size in the 3rd octet is $256 - 252 = 4$. Subnets increment by 4 (0.0, 4.0, 8.0). For the 2nd subnet:

Network ID: 172.16.4.0

Broadcast Address: 172.16.7.255

Scenario 7: The Corporate Headquarters The corporation uses 172.25.0.0/16 and needs exactly 500 usable host addresses per subnet.

- A) Required Host Bits: Using $2h- 2 \ge 500$, we need 9 host bits ($29 - 2 = 510$).

- B) Subnet Mask: Leaving 9 bits for hosts means 23 bits are for the network ($32 - 9 = 23$). The CIDR is /23. The 3rd octet uses 7 bits (11111110), resulting in a decimal mask of 255.255.254.0.

- C) Block Size: In the 3rd octet, the block size is $256 - 254 =$ 2.

Scenario 8: The Smart City Project The city uses 128.10.0.0/16 and needs 100 subnets.

- A) Subnet Mask: Using $2n\ge 100$, we borrow 7 bits ($27 = 128$). Adding 7 bits to /16 gives /23. The decimal mask is 255.255.254.0.

- B) Usable IP Addresses: A /23 mask leaves 9 host bits. $29 - 2 =$ 510 usable IP addresses.

- C) 3rd Subnet Network ID: The block size is 2 in the 3rd octet. The networks are 0.0, 2.0, 4.0. The 3rd subnet is 128.10.4.0.

Scenario 9: The University Wi-Fi Zones The university uses 172.31.0.0/16 and needs 2,000 wireless devices per subnet.

- A) Subnet Mask: Using $2h - 2 \ge 2000$, we need 11 host bits ($211 - 2 = 2046$). This leaves 21 network bits ($32 - 11 = 21$). The CIDR is /21. The 3rd octet uses 5 bits (11111000), making the decimal mask 255.255.248.0.

- B) Total Subnets: We borrowed 5 bits from the /16. $25 =$ 32 total subnets.

- C) 1st Subnet Range: The block size in the 3rd octet is $256 - 248 = 8$. The first subnet spans from 0.0 to 7.255.

Usable IP Range: 172.31.0.1 to 172.31.7.254

Scenario 10: The Manufacturing Plant The plant uses 150.20.0.0/16 and needs exactly 16 equal-sized subnets.

- A) Subnet Mask: Using $2n = 16$, we borrow 4 bits ($24 = 16$). Adding 4 bits to /16 gives /20. The 3rd octet uses 4 bits (11110000), making the decimal mask 255.255.240.0.

- B) Usable Hosts: A /20 mask leaves 12 host bits. $212- 2 = 4096 - 2 =$ 4,094 usable hosts.

- C) 5th Subnet Details: The block size is $256 - 240 = 16$ in the 3rd octet. Subnets increment by 16 (0.0, 16.0, 32.0, 48.0, 64.0). The 5th subnet is 64.0.

Network ID: 150.20.64.0

Usable IP Range: 150.20.64.1 to 150.20.79.254

Broadcast Address: 150.20.79.255